Gray Spring

- 6.6mm max compression (from datasheet)

- 17.24 deg ankle angle (from CAD)

- 452 N/mm (from datasheet)

- 62 Nm peak moment (code below)

- 206 Nm/rad peak stiffness

Yellow Spring

- 8 mm max compression (from datasheet)

- 20.51 deg ankle angle (from CAD)

- 224 N/mm (from datasheet)

- 37 Nm peak moment (code below)

- 103 Nm/rad peak stiffness

VSO Linear Cam

- Peak stiffness is 232% stiffness of yellow tango

- Peak stiffness is 116% stiffness of gray tango

Black: estimated torque-angle function for plastic cam tested at Ottobock

Yellow: yellow spring tango

Dark Gray: gray spring tango

Light gray lines: VSO at different spring support locations with linear cam